* РЭС-60 — реле с учетом дребезга контактов

* ПАСПОРТ: РС4.569.435-04, РС4.569.435-09

* рабочее напряжение (3,5...4,5) В.

.subckt PC-60 coila coilb no1 nc1 com1 no2 nc2 com2

+ PARAMS:

+ T_make = 3.5mSec ; время срабатывания

+ T_break = 1.5mSec ; время отпускания

+ T_bounce =0.5mSec ; время дребезга

+ I_pull = 60ma ; ток срабатывания

+ I_drop = 13ma ; ток отпускания

+ R_coil = 35 ; сопротивление катушки постоянному току

+ L_coil = 5mH ; индуктивность катушки

+ R_open = 100MEG ; минимальное сопротивление

* разомкнутого контакта

+ R_close = .05 ; максимальное сопротивление

* замкнутого контакта

+ Contact_match = .95 ; коэффициент времени: 1 = точно

* электрическая модель катушки

v_winding coila  a1     0

r_winding a1     a2     {R_coil}

l_winding a2     coilb  {L_coil}

* преобразователь ток—напряжение

e_cc  cc 0 value = {limit(I(v_winding),-3*I_pull,3*I_pull)}

r_cc  cc 0 1k

* моделирование гистерезиса

o_mag  cc 0 relay_1 DGTLNET=d digio_1

u_dly1 buf dpwr dgnd d cn_no1 relay_2a digio_1

u_inv1 inv dpwr dgnd d cn_nc1 relay_2a digio_1

u_dly2 buf dpwr dgnd d cn_no2 relay_2b digio_1

u_inv2 inv dpwr dgnd d cn_nc2 relay_2b digio_1

* моделирование дребезга контактов

x_no1  cn_no1 com1 no1 RELAY_CONTACT_BOUNCE

+ PARAMS:

+ T_make =  {T_make}

+ T_break=  {T_break}

+ T_bounce= {T_bounce}

+ R_open =  {R_open}

+ R_close=  {R_close}

x_nc1  cn_nc1 com1 nc1 RELAY_CONTACT_BOUNCE

+ PARAMS:

+ T_make =  {T_make}

+ T_break=  {T_break}

+ T_bounce= {T_bounce}

+ R_open =  {R_open}

+ R_close=  {R_close}

x_no2  cn_no2 com2 no2 RELAY_CONTACT_BOUNCE

+ PARAMS:

+ T_make =  {T_make*Contact_match}

+ T_break=  {T_break*Contact_match}

+ T_bounce= {T_bounce*Contact_match}

+ R_open =  {R_open}

+ R_close=  {R_close}

x_nc2  cn_nc2 com2 nc2 RELAY_CONTACT_BOUNCE

+ PARAMS:

+ T_make =  {T_make*Contact_match}

+ T_break=  {T_break*Contact_match}

+ T_bounce= {T_bounce*Contact_match}

+ R_open =  {R_open}

+ R_close=  {R_close}

.model relay_1 doutput (

+ s0name="0" s0vlo={-I_pull}   s0vhi={I_pull}

+ s1name="1" s1vlo={I_drop}    s1vhi={4*I_pull}

+ s2name="1" s2vlo={-4*I_pull} s2vhi={-I_drop}

+ timestep={T_make/1000}

+ )

.model relay_2b ugate (

+ tphlmn={T_break*Contact_match}

+ tphlmx={T_break*Contact_match}

+ tplhmn={(T_make-0.126*T_bounce)*Contact_match}

+ tplhmx={(T_make-0.126*T_bounce)*Contact_match}

+ )

.model relay_2a ugate (

+ tphlmn={T_break}   tphlmx={T_break}

+ tplhmn={(T_make-0.126*T_bounce)}

+ tplhmx={(T_make-0.126*T_bounce)}

+ )

.model digio_1 uio

.ends